- GenRef

- v1.00

- MDOS Reference guide.

- Math Library

- (C) Copyright 2004

Beery W. Miller

ALL RIGHTS RESERVED

____________________________________________________________________________________

Math - CONTENTS               ____________________________________________________________________________________

Math Overview	3

Math Details – Radix 100	4

Calling Math Functions	5

Floating Point Compare	6

Floating Point Subtract	7

Floating Point Add	8

Floating Point Multiply	9

Floating Point Divide	10

Floating Point Power	11

Floating Point ex	12

Floating Point ln(x)	13

Floating Point sqr(x)	14

Floating Point cos(x)	15

Floating Point sin(x)	16

Floating Point tan(x)	17

Floating Point atn(x)	18

Floating Point Greatest Ingeter	19

Convert Floating Point to Integer	20

Convert Integer to Floating Point	21

Convert String to Integer	22

Convert String to Floating Point	23

Convert Floating Point to String	24

____________________________________________________________________________________

MATH - OVERVIEW ____________________________________________________________________________________

All math management routines in MDOS are provided to aid a programmer in writing applications requiring math operations beyond the immediate instruction set of the TMS 9995 microprocessor.  The following math operations are supported within the operating system:

- Floating Point Compare	FCOMP

- Floating Point Subtract	FSUB

- Floating Point Add	FADD

- Floating Point Multiply	FMULT

- Floating Pont Divide	FDIV

- Floating Point Power	PWR

- Floating Point ex	EXP

- Floating Point ln(x)	LOG

- Floating Point sqr(x)	SQR

- Floating Point cos(x)	COS

- Floating Point sin(x)	SIN

- Floating Point tan(x)	TAN

- Floating Point atn(x)	ATN

- Floating Point Greatest Integer	GRI

- Convert Floating Point to Integer	CFI

- Convert Integer to Floating Point	CIF

- Convert String to Integer	CSINT

- Convert String to Floating Point	CSN

- Convert Floating Point to String	CNS

____________________________________________________________________________________

MATH Details – Radix 100           ____________________________________________________________________________________

All floating point arguments must be on an even byte boundary and the calling registers must be in PAD from >F000 to >F060.  Floating Point representation is in Radix 100 format:

0	=	00   00 xx   xx    xx   xx    xx   xx

+n	=	e0  m0 m1  m2  m3  m4  m5  m6

e0 is int(log[100](n)) + >40

m0 – m6 are numbers from >00 to >63 (0 to 99)

m0 is the most significant digit of mantissa,

m6 is the least significant digit of mantisa.

In normalized numbers, decimal is between m0 and m1

-n	is same as “n” except first word is negated … -(e0 m0)

Examples:

Decimal	  Floating Point

| 7 | >40 | >07 | >00 | >00 | >00 | >00 | >00 | >00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70 | >40 | >46 | >00 | >00 | >00 | >00 | >00 | >00 |
| 2,345,600 | >43 | >02 | >22 | >38 | >00 | >00 | >00 | >00 |
| 23,456,000 | >43 | >17 | >2D | >3C | >00 | >00 | >00 | >00 |
| 0 | >00 | >00 | >xx | >xx | >xx | >xx | >xx | >xx |
| -7 | >BF | >F9 | >00 | >00 | >00 | >00 | >00 | >00 |
| -70 | >BF | >BA | >00 | >00 | >00 | >00 | >00 | >00 |
| -2,345,600 | >BC | >FE | >22 | >38 | >00 | >00 | >00 | >00 |

____________________________________________________________________________________

CALLING MATH FUNCTIONS

____________________________________________________________________________________

The MDOS Math Library must be called from within a machine code program running as a task under MDOS. You pass arguments to the Math Library via the calling registers.

The MDOS Video Library is invoked from a machine code program when software trap number zero (XOP 0) is called with a library number of 10. The calling program's R0 must contain the 16-bit subprogram  at the time of the XOP.  The following code fragment will convert a string to a floating point number.

LI	R0,>0011	Convert String to Floating Point

LI	R1,RES	Result Pointer

LI	R2,STR1	String Pointer

LI	R3,3	3 characters to convert

XOP	@MATH,0	Access subprogram

MATH	DATA	10

STR1	TEXT	“123”

BYTE	0		Null terminated

RES     DATA 	0,0,0,0   	Floating Point Result

EVEN

In the preceding example, two hidden assumptions were made.  First it is assumed that STR1 is located on a page which is currently mapped into a memory page which has the same 16-bit address page number as its Virtual address page number (read the section on Memory Management.) The second assumption is that MATH is actually at the virtual address MATH, not in some overlay segment with a different virtual address.

Math Library

Floating Point Compare

Function	Floating Point Compare

Parameters	R0 =  >0000

R2 =  Pointer to Float 1

R3 =  Pointer to Float 2

Results	Status Register =	AG set if (Float2 > Float1)

EQ set if (Float2 = Float1)

Parameter Description

Float1	8 byte representation in Radix 100 format.

Float2	8 byte representation in Radix 100 format.

Floating Point Subtract

Function	Floating Point Subtract

Parameters	R0 =  >0001

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

R3 =  Pointer to Float 2

Results	R0 =  Error Code

R1 Pointer = Float2 – Float1

Parameter Description

Float1	8 byte representation in Radix 100 format.

Float2	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point Add

Function	Floating Point Add

Parameters	R0 =  >0002

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

R3 =  Pointer to Float 2

Results	R0 =  Error Code

R1 Pointer = Float2 + Float1

Parameter Description

Float1	8 byte representation in Radix 100 format.

Float2	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point Multiply

Function	Floating Point Multiply

Parameters	R0 =  >0003

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

R3 =  Pointer to Float 2

Results	R0 =  Error Code

R1 Pointer = Float2 * Float1

Parameter Description

Float1	8 byte representation in Radix 100 format.

Float2	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point Divide

Function	Floating Point Divide

Parameters	R0 =  >0004

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

R3 =  Pointer to Float 2

Results	R0 =  Error Code

R1 Pointer = Float2 / Float1

Parameter Description

Float1	8 byte representation in Radix 100 format.

Float2	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point Power

Function	Floating Point Power

Parameters	R0 =  >0005

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

R3 =  Pointer to Float 2

Results	R0 =  Error Code

R1 Pointer = Float2Float1

Parameter Description

Float1	8 byte representation in Radix 100 format.

Float2	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point ex

Function	Floating Point ex

Parameters	R0 =  >0006

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = eFloat1

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point ln(x)

Function	Floating Point ln(x)

Parameters	R0 =  >0007

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = ln( Float1 )

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point SQR(x)

Function	Floating Point sqr(x)

Parameters	R0 =  >0008

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = SQR( Float1 )

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point COS(x)

Function	Floating Point cos (x)

Parameters	R0 =  >0009

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = COS( Float1 )

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point SIN(x)

Function	Floating Point sin(x)

Parameters	R0 =  >000A

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = SIN( Float1 )

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point TAN(x)

Function	Floating Point tan(x)

Parameters	R0 =  >000B

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = TAN( Float1 )

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point ATN(x)

Function	Floating Point atn(x)

Parameters	R0 =  >000C

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = ATN( Float1 )

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Floating Point Greatest Integer

Function	Floating Point Greatest Integer

Parameters	R0 =  >000D

R1 =  Pointer to Result Destination

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 Pointer = || Float1 ||

Parameter Description

Float1	8 byte representation in Radix 100 format.

Result	8 byte representation in Radix 100 format.

Convert Floating Point to Integer

Function	Convert Floating Point to Integer

Parameters	R0 =  >000E

R2 =  Pointer to Float 1

Results	R0 =  Error Code

R1 =  Integer

Parameter Description

Float1	8 byte representation in Radix 100 format.

Integer	2 byte (16 bit) integer

Convert Integer to Floating Point

Function	Convert integer to Floating Point

Parameters	R0 =  >000F

R1 =  Pointer to Result Destination

R2 =  Integer

Results	R0 =  Error Code

R1 Pointer = Float( integer )

Parameter Description

Float1	8 byte representation in Radix 100 format.

Integer			2 byte (16 bit) integer

Convert String to Integer

Function	Convert String to Integer

Parameters	R0 =  >0010

R2 =  Pointer to String

Results	R0 =  Error Code

R1 =  Integer

Parameter Description

Float1	8 byte representation in Radix 100 format.

String	ASCII text string nul terminated.

Convert String to Floating Point

Function	Convert String to Floating Point

Parameters	R0 =  >0011

R1 =  Pointer to Result Destination

R2 =  Pointer to String

R3 =  String Length

Results	R0 =  Error Code

R1 Pointer = Float( String )

Parameter Description

Float	8 byte representation in Radix 100 format.

String	ASCII character text string.

Convert Float to String

Function	Convert Float to String

Parameters	R0 =  >0012

R1 =  Pointer to String

R2 =  Pointer to Float1

R3 =  opt1

Bit	0:	0 = free form (ignore opt2, opt3)

1 = fixed (opt2, opt3 are field sizes)

Bit	1:	1 for explicit sign

Bit	2:	1 to show sign of positive number as a ‘+’ instead of space

Bit	3:	1 for extended E-notation

Bit	4:	1 for extended E-notation (bit 3 must also be 1)

R4 =  if fixed format, number of places to left of decimal point,

including explicit sign.

R5 =  if fixed format, number of places to the right of decimal point,

And including decimal point

If fixed format, with exponent, R4,R5 exclude the 3 places for an exponent.

Results	R0 =  Error Code

R1 Pointer = String

Parameter Description

Float1	8 byte representation in Radix 100 format.

String	ASCII character text string.